Canon Cartridge
PG-40 (#40) and PG-50 (#50)

Refill Instructions for Black cartridge

- Instructions

1. Put the cartridge on some paper towels in an area where you can tolerate a spill.

2. Drill a hole into the dimple just below the "B".

3. Fill the syringe with 15 ml. of black ink.

4. Very S-L-O-W-L-Y inject ink into any hole.

5. When finished, put a small piece of tape over the drilled hole.

6. Replace the cartridge and run 1 - 3 cleaning cycles as per printer instructions.
Or print the following pages:
http://www.refillinstructions.com/black.htm for Black

If necessary leave cartridge in printer for up to 24 hours.

Note: Each time the cartridge is removed from the printer an "empty cartridge" message appears and a blinking amber light will come on, on the printer. Holding the paper feed button down for 3-5 sec. will allow you to override the counter.

A Refill Kit should include Ink plus the following: One 30cc Syringe with sharp needle One Thumb Drill Aluminum Tape