| Canon Cartridge PG-41 (#41) |  |
| --- | --- |
| # Refill Instructions for Color cartridge |  |

Instructions 1. Put the cartridge on some paper towels in an area where you can tolerate a spill. 2. Peal off the top label. 3. You will notice three small holes (too small for a syringe needle to get into). Drill each hole larger with a thumb drill. 4. Fill the syringe with 6 ml. of color ink. 5. Very S-L-O-W-L-Y inject ink into each chamber until the sponge full 6. When finished, replace the top label or leave it off (it will make no difference in the printing). #### 7. Replace the cartridge and run 1 - 3 cleaning cycles as per printer instructions. Or print the following page: http://www.refillinstructions.com/color.htm If necessary leave cartridge in printer for up to 24 hours. (Allows ink to flow to the print head). #### Note: Each time the cartridge is removed from the printer an "empty cartridge" message appears and a blinking amber light will come on, on the printer. Holding the paper feed button down for 3-5 sec. will allow you to override the counter. #### A Refill Kit should include Ink plus the following: #### Thumb Drill Three 10cc Syringes with sharp needle